# Rehnquist Design & Build, Inc.

11084 Gravois Industrial Ct.

St. Louis, MO 63128

January 28, 2021

Specifications

Lot 7 Belcrest Estates/13714 Belcrest Ct.

Town & Country, MO 63131

Approx. 5500 sq. ft.

TRIM

Standard trim unless noted otherwise

Cased openings per plan

4 ¼” colonial casing

5 ¼” baseboards at 2nd floor typical

7 ¼” baseboards at 1st floor typical

Solid core paneled painted interior doors, 8’ tall at 1st floor,

6’8” tall at 2nd floor

Latex on all doors and woodwork, two (2) coats

Flat latex on all walls and ceilings, two (2) coats

All drywall screwed

5/8” thick drywall at ceiling and walls

10’-0” ceilings first floor unless noted otherwise

Custom staircase

Stair parts to be Kirkwood Stair provincial red oak, painted risers and skirt boards with wood balusters and 3 ½” handrail and

box newels first and second floor

FOYER

¾” x 3 ¼” job finished select red oak flooring

5 ½” cased openings into Dining Room and Study

7 ¼” custom baseboards

5-piece crown molding

2 story ceiling

FOYER HALL

¾” x 3 ¼” job finished select red oak flooring

7 ¼” baseboards

4 ¼” casing

2-piece crown molding

DINING ROOM

¾” x 3 ¾” job finished select red oak flooring

7 ¼” baseboards

4 ¼” casing

Phone: (314) 843-3331  				     		 www.RehnquistHomes.com

5-piece crown molding

GATHERING ROOM

11’-10” ceiling height

5-piece crown molding

Hardwood flooring

7 ¼” baseboards

4 ¼” casing

Custom trimmed windows

42” Gas fireplace Heat & Glo 8000CL with natural stone surround w/custom built mantle with flush hearth

Built in bookshelves on each side of fireplace

STUDY- Painted

10’-0” ceiling height

Hardwood flooring

Panel molding on walls

7 ¼” baseboards

4 ¼” casing

3-piece crown molding

36” Gas fireplace Monessen LCUF 36” with wood and 12” marble surround

KITCHEN

Vaulted ceiling with 10”x10” beams

Hardwood flooring

4 ¼” casing

7 ¼” baseboards

Granite counter tops from cabinet/top allowance with tile backsplash

48” Sub Zero fridge/ freezer - panel front – from cabinetry/top allowance

48” Wolf dual fuel range w/ griddle and double ovens

Wolf 400 cfm int blower and Wolf 48” hood insert

24” Wolf microwave drawer – stainless steel

(2) Asko dishwashers – panel front – from cabinetry/top allowance

(2) garbage disposals – ¾ h.    p./each at main sink and secondary sink

Melamine shelves in walk-in pantry from allowance

Plumbing fixtures per allowance

Ceramic tile backsplash $25/sq.ft. installed allowance

BUTLER’S PANTRY

Cabinetry and tops from allowance

Zephyr under counter wine refrigerator

BAR

Custom cabinetry and slab granite top (from cabinet/top allowance)

24” Zephyr under counter refrigerator

(1) garbage disposal ½ h.p.

LAUNDRY – FIRST FLOOR

Built-in cabinets with drop in utility sink (from cabinet/top/plumbing fixture allowance)

Electric to dryer

Ceramic tile flooring on ½” durock - $20/ sq. ft. installed allowance

POWDER ROOM by LAUNDRY

4 ¼” casing

5 ¼” baseboards

Vanities and natural stone tops from cabinetry/top allowance

Framed mirrors over vanities allowance - $200/each

Hardwood flooring

BREAKFAST ROOM

Vaulted ceiling with 10” x 10” beams

Hardwood flooring same as kitchen

4 ¼” casing

7 ¼” baseboards

GUEST SUITE – 1ST FLOOR

5 ¼” crown in bedroom

4 ¼” case

5 ¼” base

Carpet - $5/sq. ft. installed allowance

MASTER SUITE

11’-6” Coffered ceiling

Carpeting per allowance

4 ¼” colonial casing

7 ¼” baseboards

3-piece crown molding at each level of coffer

Sound insulate walls & ceiling

MASTER BATH

Natural stone floor on ½” durock  - $ 25/sq.ft. installed allowance

4 ¼” colonial casing

7 ¼” baseboards

3-piece crown molding

Vanities with slab natural stone top from allowance

(2) Undermount sinks

Freestanding tub

Natural stone tiled shower surround over ½” durock with marble

tiled shower base on concrete and sand - $ 25/sq.ft. installed allowance

Heated floor system

3/8” shower glass surround - frameless

¼” plate mirror with custom casing

All bathrooms insulated for sound at walls and all drain stacks

Plumbing fixtures per allowance

Separate tankless water heater servicing just Master Bath 199,000 BTU

POWDER ROOM - Entry

Pedestal sink

Hardwood flooring

7 ¼” baseboards

2 Piece crown molding

UPSTAIRS HALLS

1-piece crown

Hardwood flooring

## UPSTAIRS BEDROOMS

9’-0” ceilings

Carpeting – allowance at $5/ per sq. ft. installed

4 ¼” colonial casing

5 ¼” colonial base

5 ¼” one-piece crown molding in flat ceiling areas and halls

## BATHROOMS (incl. 1st floor Guest)

Ceramic tile flooring on ½” durock - $20/ sq. ft. installed allowance

4 ¼” casing

5 ¼” baseboards

Vanities and natural stone tops from cabinetry/top allowance

Framed mirrors over vanities allowance - $200/each

Ceramic tile tub and/or shower surrounds over ½” durock

All bathroom walls and drain stacks insulated for sound

Plumbing fixtures per selections

Ceramic tile shower bases on cement

## ELECTRICAL

See house plans for all electrical information

*Lighting - $25,000 lighting allowance

## ENERGY AND SECURITY

# R – 40 insulation in ceiling; approximately R – 21 full thick wall insulation with sheathing

Complete Tyvek house wrap at all exterior walls

Infiltration control system

Fiberglass sill sealer on foundation plates

Floor joists bank board insulation R – 15

Trane 13 SEER high air conditioning units

Trane 95+% high efficiency furnaces

Three (3) zoned heating and air conditioning with Aprilaire 760 humidifiers

Automated timed thermostat – Maple Chase

Mechanical return air at every room other than bathrooms and laundry room

Outside air access for first floor fireplace

Fully insulated, drywalled and painted garages

Metal raised panel, insulated garage doors

Emtek door hardware

Deadbolt keyed locks for all hinged exterior doors

Smoke detector on each floor and all bedrooms

Carbon monoxide detectors on each floor

Pella Lifestyle Series windows insulated low E argon gas glass, clad exterior

Pella Steel insulated exterior doors with thermo paned and tempered glass and deadbolts

Security System: Ademco alarm keypad, Ademco siren, Ademco Vista 20 Panel, cellular backup, (4) camera prewire.

(4) prewire 1-pair stereo audio zone, prewire 5.1 surround with inwall sub

TV prewires located in office, kitchen, master bedroom, master bath, gathering room, all bedrooms, bonus room,  breakfast room, veranda, network prewire

Prewire security cameras

## QUALITY AND CONVENIENCE

# All drywall screwed

All framing members are 16” on center with kiln dried and graded lumber used throughout

12” floor trusses 16” on center

All lumber in contract with concrete to be wolmanized treated lumber

¾” tongue and groove sub-floor, glued and nailed

½” fir roof sheathing

400-amp electrical service with sub panel

200,000 BTU tankless water heater for master suite

(1) 75-gallon fast recovery gas water heater

Internet prewire

Prewired for speakers in study, dining room, kitchen and veranda

Prewired for surround sound in gathering room

Whole house closet and pantry melamine shelving allowance installed - $23,000

Insect screens on all operable windows (except garage)

Carpet with 9/16” pad in bedrooms, closets, stairways and hall closets

Whole house carpet allowance installed $5/sq. ft.

Beam Central Vac – rough in only

2” copper water service

10-year home owner warranty

## LOWER LEVEL

Finished to painted drywall

All plumbing, electric and mechanical rough included

4th mechanical zone

Finish electric and plumbing labor included

All ceiling areas to be sound insulated

Ceiling height to vary from 9’-0” to 9’-10”

Staircase with ½ wall – stair parts to match first floor stairs

50 CFM exhaust fan for bathroom

Bathroom to be sound insulated

(2) TV outlets pre-wired

(4) speakers pre-wired at entertainment area

## OUTSIDE, BASEMENT AND GARAGE

Brick, and Hardie Stucco front, brick sides and rear.  Masonry siding rear & sides per plan

30-year architectural shingle, synthetic underlayment, ice & water shield at entire perimeter and valleys

Prefinished vinyl enclosed soffits and aluminum wrapped fascia

Prefinished aluminum 6” gutters

4” drain tile, gravel and filter fabric around the exterior perimeter of house and interior drain tile with clean rock around the perimeter with both drained out to day light

Complete waterproofed basement – break and patch all exterior below grade tie rods and spray with Mulseal Bituminous

asphalt

4” thick concrete floor at basement

10’-0” tall foundation 12” thick (3 pr #4 walls and 1 pr #4 at footing)

Asphalt drive 12’0” wide 8” rock base with 2” – 2 ½” C asphalt drive

20’ Wide circle Drive

Rear veranda with stained V-groove ceiling and structural concrete floor with stamped flagstone

Deck with and aluminum rail

(4) Car side entry garage with full basement

Lower level garage/storage below first floor garage w/ 9’-0 x 8’-0 overhead door with opener and French doors

Garage staircase to lower level garage

Five (5) outside faucets

All disturbed areas graded and sodded

Lawn fully irrigated

Front landscaping

Landscaped berms along Mason Rd. with 10’ - 12’ Norway Spruces and various deciduous trees

Two (2) automatic garage door openers and remote controls

2” x 8” fascia and 1” x 6” freeze board at all elevations

Termite control

1.95 acre home site with rear walkout

Pre-Construction Price: $ 2,458,000.00